Azure RTOS 
Samples for

STM32L4+-DISCO IoT Node using IAR

User Guide

Published: September 2020

For the latest information, please see 
azure.com/rtos

This document is provided “as-is”. Information and views expressed in this document, including URL and other Internet Web site references, may change without notice.

This document does not provide you with any legal rights to any intellectual property in any Microsoft product. You may copy and use this document for your internal, reference purposes.

Azure RTOS provides OEMs with components to secure communication and to create code and data isolation using underlying MCU/MPU hardware protection mechanisms. It is ultimately the responsibility of the device builder to ensure the device fully meets the evolving security requirements associated with its specific use case.

FileX supports the Microsoft exFAT file system format. Your use of exFAT technology in your products requires a separate license from Microsoft. Please see the following link for further details on exFAT licensing:

https://www.microsoft.com/en-us/legal/intellectualproperty/mtl/exfat-licensing.aspx

© 2020 Microsoft. All rights reserved.

Microsoft Azure RTOS, Azure RTOS FileX, Azure RTOS GUIX, Azure RTOS GUIX Studio, Azure RTOS NetX, Azure RTOS NetX Duo, Azure RTOS ThreadX, Azure RTOS TraceX, Azure RTOS Trace, event-chaining, picokernel, and preemption-threshold are trademarks of the Microsoft group of companies. All other trademarks are property of their respective owners.

Revision 6.1

Table of Contents

Overview	1

Getting Started	4

Sample Descriptions	6

Azure IoT HUB Connectivity Sample	Error! Bookmark not defined.

ThreadX Sample	7

NetX Duo Simple Ping Sample	8

# Overview

STM32L4S5I-DISCO IoT Node board

Azure RTOS Samples for each component (Azure connectivity, ThreadX and NetX Duo) are designed to run on the STM32L4S5I-DISCO IoT Node “out-of-the-box.” Each sample project is described later in this document along with links to further information as necessary.

All samples are designed to run using the IAR Embedded Workbench for ARM (EWARM) 8.50.1 or later, with the on-board ST-LINK debugger (Debug USB port). The default factory jumper selections are assumed. A 30-day free trial of IAR’s EWARM development tools can be downloaded from this page:

https://www.iar.com/iar-embedded-workbench/#!?architecture=Arm

The sample distribution zip file has following organization:

The root directory contains the azure_rtos.eww workspace as well as following sub-folders:

Folder		Contents

iar  	Contains the following sub-folders as well as the azure_rtos.eww workspace

common_hardware_code	Contains common code for STM32L4S5I-DISCO board

docs	Contains user guides and supporting documentation

netxduo	Contains NetX Duo and NetX Secure source code

sample_azure_iot_embedded_sdk

Sample project to connect to Azure IoT Hub using Azure IoT Middleware for Azure RTOS

sample_azure_iot_embedded_sdk_pnp

Sample project to connect to Azure IoT Hub using Azure IoT Middleware for Azure RTOS via IoT Plug and Play

sample_pnp_temperature_controller

Sample project with IoT Plug and Play using multiple components

sample_netx_duo_ping	Contains NetX Duo ping sample project

sample_threadx	Contains ThreadX sample project

stm32l4xx_lib	Contains STM32L4+ drivers

threadx	Contains ThreadX source code

# Getting Started

- Unpack the sample zip file into a folder of your choice, we recommend:

		C:\azure_rtos\b-l4s5i-iot01a\iar

NOTE: If you unzip the IAR project in a very long path, it might not open properly.

- Open the azure_rtos.eww EWARM Workspace, using IAR EWARM.

- Select the desired sample project in the IAR workspace. There is a menu of available reference project tabs in the Project Pane of the EWARM IDE (as shown below). Click on the desired sample project and make the project active by selecting Active Project via a right-click.

 

The sample_threadx project is shown above as the currently active project.

- Configure the WIFI_SSID and WIFI_PASSWORD by right click on active project, select Options > C/C++ Compiler > Preprocessor. Replace the values for your WiFi to be used.

- Select Project > Batch Build and choose build_all and select Make to build all projects. You will observe compilation and linking of all sample projects.

- Select Download and Debug to download and start execution of the project. By default, execution stops at a breakpoint set at main.

- Select Go to start execution of the demonstration. Please review the sample descriptions later in this guide for additional setup and expected behavior.

The sample project can also run on STM32L475-DISCO IoT Node with just a few configurations:

- Right click on active project, select Options > General Options. In Target tab, change the device to ST STM32L475VG.

- In Options > Linker, replace the configuration file with common_hardware_code/ stm32l475xx_flash.icf

# Sample Descriptions

## Azure RTOS IoT Embedded SDK Samples

Please see the Azure_RTOS_STM32L4+-DISCO_Azure_IoT_Embedded_SDK_For_IAR.pdf for a detailed description of the sample as well as a step-by-step set of instructions on how to connect to Azure IoT via the Azure IoT Embedded C SDK and Azure IoT Middleware for Azure RTOS.

## ThreadX Sample

This sample is the standard 8-thread ThreadX example, that illustrates the use of the main ThreadX services, including threads, message queues, timers, semaphores, byte memory pools, block memory pools, event flag groups, and mutexes. This demonstration is fully described, including a source code listing, in Chapter 6 of the Azure_RTOS_ThreadX_User_ Guide.pdf.

To run the ThreadX Sample project, simply follow these steps (assuming the azure_rtos.eww workspace is already open):

- Click on the sample_threadx project and make the project active by selecting Active Project via a right-click.

- Select Make button to build the Active Project selected. You will observe compilation and linking of the selected sample project.

- Select Download and Debug to download and start execution of the demonstration. The sample will initially stop at main. Select another Go to execute the sample.

After hitting Break the IAR debugger screen shot above shows various counters incremented by the ThreadX sample as each of the main components of the ThreadX are exercised.

To learn more about Azure RTOS ThreadX, view https://docs.microsoft.com/azure/rtos/threadx/.

## NetX Duo Simple Ping Sample

This sample project illustrates the setup and use of NetX Duo IPv4/IPv6 TCP/IP stack via ping from another node on the local network. By default, this demonstration requests an IP Address via DHCP, and displays the status and assigned IP Address via Terminal I/O in the IAR debugger. In addition, the retrieved IP address is displayed in the watch window of the IAR debugger.

To run the NetX Duo Ping Sample project, simply follow these steps (assuming the azure_rtos.eww workspace is already open):

- Click on the sample_netx_duo_ping project and make the project active by selecting Active Project via a right-click.

- Select Make button to build the Active Project selected. You will observe compilation and linking of the selected sample project. Note: This sample is Ethernet based and therefore assumes an Ethernet cable is connected to the Ethernet connector on the board.

- Select Download and Debug to download and start execution of the demonstration. The sample will initially stop at main. Select another Go to execute the sample. At this point, you should observe the IP address assigned via DHCP in the Terminal I/O window.

The example above shows that the assigned IP address of the STM32L4S5I-DISCO IoT Node board is 10.10.10.106.  When the demonstration is running it can be pinged by any machine on the network. The following is an example of a ping from a Windows machine on the same local netword (using the DOS command window):

To learn more about Azure RTOS NetX Duo, view https://docs.microsoft.com/azure/rtos/netx/.